1.

set ns [new Simulator]

set nf [open out.nam w]

$ns namtrace-all $nf

set tf [open out.tr w]

$ns trace-all $tf

proc finish {} {

global ns nf tf

$ns flush-trace

close $nf

close $tf

exec nam out.nam &

exit 0

}

set n0 [$ns node]

set n1 [$ns node]

set n2 [$ns node]

set n3 [$ns node]

$ns duplex-link $n0 $n2 100Mb 10ms DropTail

$ns duplex-link $n1 $n2 1Mb 5ms DropTail

$ns duplex-link $n2 $n3 1Mb 5ms DropTail

$ns duplex-link $n1 $n3 1Mb 5ms DropTail

$ns queue-limit $n0 $n2 50

$ns queue-limit $n1 $n2 50

$ns queue-limit $n2 $n3 50

$ns queue-limit $n1 $n3 50

set udp1 [new Agent/UDP]

$ns attach-agent $n0 $udp1

set null [new Agent/Null]

$ns attach-agent $n3 $null

$ns connect $udp1 $null

set cbr1 [new Application/Traffic/CBR]

$cbr1 set packet-size 200

$cbr1 set interval 0.005

$cbr1 attach-agent $udp1

$ns at 0.5 "$cbr1 start"

$ns at 5.0 "$cbr1 stop"

$ns at 5.5 "finish"

$ns run

2.

set ns [new Simulator]

$ns color 1 violet

set nf [open out.nam w]

$ns namtrace-all $nf

set tf [open out.tr w]

$ns trace-all $tf

proc finish {} {

global ns nf tf

$ns flush-trace

close $nf

close $tf

exec nam out.nam &

exit 0

}

set n1 [$ns node]

set n2 [$ns node]

set n3 [$ns node]

set n4 [$ns node]

set n5 [$ns node]

set n6 [$ns node]

$ns duplex-link $n1 $n5 0.011Mb 10ms DropTail

$ns duplex-link $n2 $n3 1Mb 5ms DropTail

$ns duplex-link $n2 $n4 1Mb 5ms DropTail

$ns duplex-link $n1 $n6 1Mb 5ms DropTail

$ns duplex-link $n3 $n6 1Mb 5ms DropTail

$ns duplex-link-op $n1 $n5 color red

$ns duplex-link-op $n2 $n3 color blue

$ns duplex-link-op $n1 $n5 orient left

$ns duplex-link-op $n2 $n3 orient right

$ns queue-limit $n1 $n5 50

$ns queue-limit $n2 $n3 50

$ns queue-limit $n2 $n4 50

$ns queue-limit $n1 $n6 50

$ns queue-limit $n3 $n6 50

$n1 color blue

$n2 color red

$n1 shape circle

$n2 shape box

set tcp1 [new Agent/TCP]

$ns attach-agent $n3 $tcp1

$tcp1 set fid_ 1

set tcpsink0 [new Agent/TCPSink]

$ns attach-agent $n5 $tcpsink0

$ns connect $tcp1 $tcpsink0

set tcp2 [new Agent/TCP]

$ns attach-agent $n4 $tcp2

set tcpsink1 [new Agent/TCPSink]

$ns attach-agent $n6 $tcpsink1

$ns connect $tcp2 $tcpsink1

set ftp0 [new Application/FTP]

set telnet0 [new Application/Telnet]

$ftp0 set packet-size 200

$ftp0 set interval 0.005

$ftp0 attach-agent $tcp1

$ns at 0.5 "$ftp0 start"

$ns at 4.5 "$ftp0 stop"

$telnet0 set packet-size 200

$telnet0 set interval 0.005

$telnet0 attach-agent $tcp2

$ns at 0.5 "$telnet0 start"

$ns at 4.5 "$telnet0 stop"

$ns at 5.5 "finish"

$ns run

3.

set ns [new Simulator]

set nf [open out.nam w]

$ns namtrace-all $nf

set tf [open out.tr w]

$ns trace-all $tf

$ns color 1 red

$ns color 2 brown

proc finish {} {

global ns nf tf

$ns flush-trace

close $nf

close $tf

exec nam out.nam &

exit 0

}

set n0 [$ns node]

set n1 [$ns node]

set n2 [$ns node]

set n3 [$ns node]

set n4 [$ns node]

set n5 [$ns node]

set n6 [$ns node]

set n7 [$ns node]

set n8 [$ns node]

set n9 [$ns node]

$ns make-lan "$n0 $n1 $n2 $n3 $n4 $n5 $n6 $n7 $n8 $n9" 0.1Mb 0.1ms LL Queue/DropTail Mac/802_3

set tcp0 [new Agent/TCP]

$ns attach-agent $n3 $tcp0

$tcp0 set fid_ 1

set tcpsink0 [new Agent/TCPSink]

$ns attach-agent $n5 $tcpsink0

$ns connect $tcp0 $tcpsink0

set telnet0 [new Application/Telnet]

$telnet0 set packet-size 500

$telnet0 set interval 0.005

$telnet0 attach-agent $tcp0

$ns at 0.5 "$telnet0 start"

$ns at 2.0 "$telnet0 stop"

set tcp1 [new Agent/TCP]

$ns attach-agent $n3 $tcp1

$tcp1 set fid_ 2

set tcpsink1 [new Agent/TCPSink]

$ns attach-agent $n5 $tcpsink1

$ns connect $tcp1 $tcpsink1

set telnet1 [new Application/Telnet]

$telnet1 set packet-size 500

$telnet1 set interval 0.005

$telnet1 attach-agent $tcp1

$ns at 0.5 "$telnet1 start"

$ns at 2.0 "$telnet1 stop"

$ns at 5.0 "finish"

$ns run

4.

set ns [new Simulator]

$ns color 1 brown

$ns color 2 green

set nf [open out.nam w]

$ns namtrace-all $nf

set tf [open out.tr w]

$ns trace-all $tf

proc finish {} {

global ns nf tf

$ns flush-trace

close $nf

close $tf

exec nam out.nam &

exit (0)

}

set n0 [$ns node]

set n1 [$ns node]

set n2 [$ns node]

set n3 [$ns node]

set n4 [$ns node]

set n5 [$ns node]

set n6 [$ns node]

set n7 [$ns node]

set n8 [$ns node]

set n9 [$ns node]

set n10 [$ns node]

set n11 [$ns node]

$ns make-lan "$n0 $n1 $n2 $n3 $n4 $n5 $n6 $n7 $n8 $n9 $n10 $n11" 0.22Mb 10ms LL

Queue/DropTail Mac/802_3

$n0 color red

$n1 color red

$n2 color red

$n3 color red

$n4 color blue

$n5 color blue

$n6 color blue

$n7 color blue

$n8 color blue

$n9 color purple

$n10 color purple

$n11 color purple

set udp1 [new Agent/UDP]

$ns attach-agent $n0 $udp1

set null1 [new Agent/Null]

$ns attach-agent $n3 $null1

$ns connect $udp1 $null1

set tcp1 [new Agent/TCP]

$ns attach-agent $n4 $tcp1

$tcp1 set fid_ 1

set tcpsink0 [new Agent/TCPSink]

$ns attach-agent $n8 $tcpsink0

$ns connect $tcp1 $tcpsink0

set tcp2 [new Agent/TCP]

$ns attach-agent $n9 $tcp2

$tcp2 set fid_ 2

set tcpsink1 [new Agent/TCPSink]

$ns attach-agent $n11 $tcpsink1

$ns connect $tcp2 $tcpsink1

set cbr1 [new Application/Traffic/CBR]

$cbr1 set packet-size 200

$cbr1 set interval 0.005

$cbr1 attach-agent $udp1

$ns at 0.5 "$cbr1 start"

$ns at 4.0 "$cbr1 stop"

set ftp0 [new Application/FTP]

$ftp0 set packet-size 200

$ftp0 set interval 0.005

$ftp0 attach-agent $tcp1

$ns at 0.5 "$ftp0 start"

$ns at 4.0 "$ftp0 stop"

set telnet0 [new Application/Telnet]

$telnet0 set packet-size 200

$telnet0 set interval 0.005

$telnet0 attach-agent $tcp2

$ns at 0.5 "$telnet0 start"

$ns at 4.5 "$telnet0 stop"

$ns at 5.0 "finish"

$ns run

5.

set ns [new Simulator]

set nf [open out.nam w]

$ns namtrace-all $nf

set tf [open out.tr w]

$ns trace-all $tf

proc finish {} {

global ns nf tf

$ns flush-trace

close $nf

close $tf

exec nam out.nam &

exit 0

}

Agent/Ping instproc recv {from rtt} {

$self instvar node_

puts "node [$node_ id] has recieved ping answer $from with round trip time $rtt ms"

}

set n1 [$ns node]

set n2 [$ns node]

set n3 [$ns node]

set n4 [$ns node]

set n5 [$ns node]

set n6 [$ns node]

$ns duplex-link $n1 $n5 1Mb 10ms DropTail

$ns duplex-link $n2 $n3 1Mb 5ms DropTail

$ns duplex-link $n2 $n4 1Mb 5ms DropTail

$ns duplex-link $n1 $n6 1Mb 5ms DropTail

$ns duplex-link $n3 $n6 1Mb 5ms DropTail

$ns queue-limit $n1 $n5 50

$ns queue-limit $n2 $n3 50

$ns queue-limit $n2 $n4 50

$ns queue-limit $n1 $n6 50

$ns queue-limit $n3 $n6 50

set p1 [new Agent/Ping]

$ns attach-agent $n1 $p1

set p2 [new Agent/Ping]

$ns attach-agent $n3 $p2

$ns connect $p1 $p2

$ns at 0.2 "$p1 send"

$ns at 0.6 "$p2 send"

$ns at 0.4 "$p1 send"

$ns at 0.8 "$p2 send"

$ns at 1.0 "finish"

$ns run

6.

set ns [new Simulator]

set nf [open out.nam w]

$ns namtrace-all $nf

set tf [open out.tr w]

$ns trace-all $tf

proc finish {} {

global ns nf tf

$ns flush-trace

close $nf

close $tf

exec nam out.nam &

exit 0

}

set numnodes 5

for { set i 0 } {$i<$numnodes} {incr i} {

set nodes($i) [$ns node]

}

for { set i 0 } {$i<$numnodes} {incr i} {

set j [expr {($i+1)%$numnodes}]

$ns duplex-link $nodes($i) $nodes($j) 1Mb 10ms DropTail

}

Agent/Ping instproc recv {from rtt} {

$self instvar node_

puts "node [$node_ id] has received ping answer $from with round trip time $rtt ms"

}

set p1 [new Agent/Ping]

$ns attach-agent $nodes(0) $p1

set p2 [new Agent/Ping]

$ns attach-agent $nodes(2) $p2

$ns connect $p1 $p2

$ns at 0.2 "$p1 send"

$ns at 0.6 "$p2 send"

$ns at 0.4 "$p1 send"

$ns at 0.8 "$p2 send"

$ns at 1.0 "finish"

$ns run

7.

set ns [new Simulator]

set nf [open out.nam w]

$ns namtrace-all $nf

set tf [open out.tr w]

$ns trace-all $tf

$ns color 1 red

$ns color 2 blue

proc finish {} {

global ns nf tf

$ns flush-trace

close $nf

close $tf

exec nam out.nam &

exit 0

}

set numnodes 7

for { set i 0 } {$i<$numnodes} {incr i} {

set n($i) [$ns node]

}

for { set i 1 } {$i<7} {incr i} {

$ns duplex-link $n(0) $n($i) 1Mb 10ms DropTail

}

$ns duplex-link-op $n(0) $n(1) orient left-up

$ns duplex-link-op $n(0) $n(2) orient right-up

$ns duplex-link-op $n(0) $n(3) orient left

$ns duplex-link-op $n(0) $n(4) orient left-down

$ns duplex-link-op $n(0) $n(5) orient right

$ns duplex-link-op $n(0) $n(6) orient right-down

set udp0 [new Agent/UDP]

$ns attach-agent $n(0) $udp0

$udp0 set fid_ 1

set null1 [new Agent/Null]

$ns attach-agent $n(2) $null1

$ns connect $udp0 $null1

set cbr0 [new Application/Traffic/CBR]

$cbr0 set packet-size 500

$cbr0 set interval 0.005

$cbr0 attach-agent $udp0

$ns at 0.3 "$cbr0 start"

$ns at 1.5 "$cbr0 stop"

set tcp0 [new Agent/TCP]

$ns attach-agent $n(3) $tcp0

$tcp0 set fid_ 2

set tcpsink0 [new Agent/TCPSink]

$ns attach-agent $n(5) $tcpsink0

$ns connect $tcp0 $tcpsink0

set ftp0 [new Application/FTP]

$ftp0 set packet-size 500

$ftp0 set interval 0.01

$ftp0 attach-agent $tcp0

$ns at 0.5 "$ftp0 start"

$ns at 2.0 "$ftp0 stop"

$ns at 2.5 "finish"

$ns run